# COSC 3327

# Dr. Tim McGuire

# EXAM #1

# Sample

- Draw the gate diagram symbols for

  - a NOT gate

  - an XNOR gate

  - a NAND gate

- Draw the transistor diagram for an inverter (NOT gate.)

- Define

  - combinational logic

    - Combinational logic is: outputs are dependent only on the current inputs.

    - Ex:

      - Adder

      - multiplexor

  - sequential logic

    - Sequential logic is:  the output is dependent on the current inputs and past inputs

      - Ex.

        - Vending machine change. Gumball machine

        - Memory in a computer

        - RS Flip-flop

          - Uses nor gates

          - Hooked up in a feedback circuit.

          - S – qbar

          - R – q

- Consider the Boolean logic expression: F = (AB + C) + (B  AC̅).  Draw a circuit diagram based directly on the formula using AND, OR, XOR, and NOT gates.

- For the above Boolean expression, construct a K-map, minimize the circuit, and give the sum-of-products form of the minimized expression.

- Show that the following gates can be implemented entirely using only 2-input NAND gates:

  - NOT

  - 2-input OR

  - 2-input AND

- A 1-bit half adder has the following truth table. Construct an appropriate circuit diagram:

  	A	B	S	C
 	0	0	0	0
 	0	1	1	0
 	1	0	1	0
 	1	1	0	1

- Draw the gate-level logic diagram for a 4-to-1 MUX, a circuit in which one of four data input lines are connected to its single output, depending on the state of its two control lines.

- You have been constructing a circuit and discover that you need one additional AND gate, but all you have is a chip with 2-to-1 MUXs.  Show how to connect a 2-to-1 MUX to serve as an AND gate. (Assume you have access to Vcc and Ground, in addition to your logic inputs.)

                                       D0
                                                            F
                                                                        
                                       D1     A

- Draw the gate-level logic diagram for an RS flip-flop.  Label all inputs and outputs correctly (R, S, Q, and Q̅.)

- Draw the logic gate diagram for a clocked D flip-flop. You may use a block diagram for an RS flip-flop in your solution.

- Convert 11111101 to decimal treating it as: Do in sleep

  - unsigned

  - 1’s complement

  - 2’s complement

  - sign-magnitude

- The first 32 bits of every Java class file are CAFE BABEh. Convert this to 32-bit binary, then to octal.

- Convert 429 (base 9) to base 3:

- Convert the following decimal numbers to IEEE single-precision floating-point numbers.  Report the results as hexadecimal numbers.  (You need not extend the calculation of the mantissa value beyond its most significant 8 bits.)

  - 8.25

  - –4.5

    - -4.510 = -100 .12

- Convert the following IEEE single-precision floating-point numbers to their base-10 values.

  - 3F60 0000h

  - BF90 0000h

- What it the smallest normalized positive IEEE single-precision floating-point number can you represent? Explain.

  - The smallest stored value = 1; so you’d have to subtract the bias of 126. So the fraction is 1.0 ………. ?

- Translate the following into MIPS Assembly language. A = a-b+c-d. assume they are in regs $s1-$s4

- The MIPS processor  has no PUSH or POP opcodes.  Explain how processor performs the equivalent operations.

  - ?????

- Moore's Law state that computing power doubles every 18 months for the same price.  As of  October 2009, a computer scientist at NASA has just built a cluster of 128 “typical” desktop machines that can solve a mathematical problem in 16 hours.  On what date will our scientist be able to build a cluster of 16 “typical” desktop machines that will solve the same problem in 4 hours?

  - Firstly, every machine has to be 32x faster

  - So 32 = 25

  - 5 “doublings”

  - 5x 1.5 years – 7.5 years

    - therefore April of 2017 he should expect to do it.

- We wish to compare the CPU performance of two computers, A and B, when executing program P. We know that the processors in A and B both implement the same instruction set architecture. We can determine the following information for A and B:

(1) The Instruction Count (number of instructions executed) for Program P

(2) The Clocks Per Instruction (CPI) for Program P

(3) The Clock Rate

Which of these must be determined for both A and B in order to make a valid comparison?

- CPI and clock rate can be different.

- A 1.6GHZ machine takes 8 clock cycles for every instruction. You wish to make sure that your function completes in less than 2 microseconds. To how many instructions should you limit your function?

  - CPI = 8;

  - # instructions = (using dimensional analysis)

- Consider a special floating-point representation that is the same as IEEE single and double precision (with the largest exponent value representing infinity and NaN), except that the exponent field is 3 bits and the significand field is 4 bits, for a total of 8 bits (once we include the sign bit). With this representation:

  - What is the value in base 10 of the largest representable positive number? (infinity and NaN are not numbers.)

  - What is the value in base 10 of the smallest representable positive number (not zero)? (infinity and NaN are not numbers. You may represent the result as a fraction rather than as a decimal.)

  - How many numbers total can be represented? (infinity and NaN are not numbers.) 193 numbers